ETSI TS 103 920-2 V1.1.1 (2024-10)

TECHNICAL SPECIFICATION

TECHNICAL SPECIFICATION

Core Network and Interoperability Testing (INT);

5G NGAP Conformance Testing for the N2 interface;

(3GPPTM Release 16);

Part 2: Test Suite Structure (TSS) and Test Purposes (TP)

2 V1.1.1 (2024-10)

Reference

DTS/INT-00198

Keywords

conformance, NGAP, TSS&TP

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - APE 7112B

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° w061004871

Important notice

The present document can be downloaded from the ETSI Search & Browse Standards application.

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the prevailing version of an ETSI deliverable is the one made publicly available in PDF format on ETSI deliver.

Users should be aware that the present document may be revised or have its status changed,  this information is available in the Milestones listing.

If you find errors in the present document, please send your comments to the relevant service listed under Committee Support Staff.

If you find a security vulnerability in the present document, please report it through our  Coordinated Vulnerability Disclosure (CVD) program.

Notice of disclaimer & limitation of liability

The information provided in the present deliverable is directed solely to professionals who have the appropriate degree of experience to understand and interpret its content in accordance with generally accepted engineering or  other professional standard and applicable regulations.

No recommendation as to products and services or vendors is made or should be implied.

No representation or warranty is made that this deliverable is technically accurate or sufficient or conforms to any law and/or governmental rule and/or regulation and further, no representation or warranty is made of merchantability or fitness for any particular purpose or against infringement of intellectual property rights.

In no event shall ETSI be held liable for loss of profits or any other incidental or consequential damages.

Any software contained in this deliverable is provided "AS IS" with no warranties, express or implied, including but not limited to, the warranties of merchantability, fitness for a particular purpose and non-infringement of intellectual property

rights and ETSI shall not be held liable in any event for any damages whatsoever (including, without limitation, damages for loss of profits, business interruption, loss of information, or any other pecuniary loss) arising out of or related to the use of or inability to use the software.

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.

The content of the PDF version shall not be modified without the written authorization of ETSI.

The copyright and the foregoing restriction extend to reproduction in all media.

© ETSI 2024.

All rights reserved.

Contents

Intellectual Property Rights ................................................................................................................................ 5

Foreword ............................................................................................................................................................. 5

Modal verbs terminology .................................................................................................................................... 5

- Scope ........................................................................................................................................................ 6

- References ................................................................................................................................................ 6

  - Normative references ......................................................................................................................................... 6

  - Informative references ........................................................................................................................................ 6

- Definition of terms, symbols and abbreviations ....................................................................................... 7

  - Terms .................................................................................................................................................................. 7

  - Symbols .............................................................................................................................................................. 7

  - Abbreviations ..................................................................................................................................................... 7

- Test configurations ................................................................................................................................... 7

  - Introduction ........................................................................................................................................................ 7

  - Test configuration using the N2 interface .......................................................................................................... 7

  - Network infrastructure ....................................................................................................................................... 9

- Test Suite Structure (TSS) and Test Purposes (TP) ................................................................................. 9

  - Test Suite Structure ............................................................................................................................................ 9

    - TP naming convention .................................................................................................................................. 9

    - Test strategy ................................................................................................................................................ 10 5.1.3 TP structure ................................................................................................................................................. 10

  - Test Purposes .................................................................................................................................................... 11

    - PICS references .......................................................................................................................................... 11

    - N2 interface ................................................................................................................................................ 11

      - gNB Role .............................................................................................................................................. 11

        - Test selection ................................................................................................................................... 11

        - PDU Session Management Procedures ........................................................................................... 11

        - UE Context Management Procedures .............................................................................................. 17

        - UE Mobility Management Procedures ............................................................................................ 26

        - Paging Procedures ........................................................................................................................... 35

        - Transport of NAS Messages Procedures ......................................................................................... 35 5.2.2.1.7 Interface Management Procedures .................................................................................................. 37

5.2.2.1.8 Configuration Transfer Procedure ................................................................................................... 39 5.2.2.1.9 Warning Message Transmission Procedures ................................................................................... 40

        - NRPPa Transport Procedures .......................................................................................................... 42

        - Trace Procedures ............................................................................................................................. 42 5.2.2.1.12 Location Reporting Procedures ....................................................................................................... 44

        - UE TNLA Binding Procedures ........................................................................................................ 45

        - UE Radio Capability Management Procedures ............................................................................... 45

        - Data Usage Reporting Procedures ................................................................................................... 46 5.2.2.1.16 RIM Information Transfer Procedures ............................................................................................ 46 5.2.2.2 AMF Role ............................................................................................................................................. 47

        - Test selection ................................................................................................................................... 47

        - PDU Session Management Procedures ........................................................................................... 47

        - UE Context Management Procedures .............................................................................................. 49

        - UE Mobility Management ............................................................................................................... 54

        - Paging Procedures ........................................................................................................................... 59

        - Transport of NAS Messages Procedures ......................................................................................... 60

        - Interface Management Procedures .................................................................................................. 61

        - Configuration Transfer Procedure ................................................................................................... 67

        - Warning Message Transmission Procedures ................................................................................... 67

        - NRPPa Transport ............................................................................................................................. 68 5.2.2.2.11 Trace Procedures ............................................................................................................................. 69 5.2.2.2.12 Location Reporting .......................................................................................................................... 69

5.2.2.2.13 	UE TNLA Binding Procedures ........................................................................................................ 70

2 V1.1.1 (2024-10)

        - UE Radio Capability Management .................................................................................................. 70

        - Data Usage Reporting Procedures ................................................................................................... 71

        - RIM Information Transfer Procedures ............................................................................................ 71

Annex A (normative): 	TDL-TO source files ...................................................................................... 72

History .............................................................................................................................................................. 73

# Intellectual Property Rights

Essential patents

IPRs essential or potentially essential to normative deliverables may have been declared to ETSI. The declarations pertaining to these essential IPRs, if any, are publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org/).

Pursuant to the ETSI Directives including the ETSI IPR Policy, no investigation regarding the essentiality of IPRs, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

Trademarks

The present document may include trademarks and/or tradenames which are asserted and/or registered by their owners. ETSI claims no ownership of these except for any which are indicated as being the property of ETSI, and conveys no right to use or reproduce any trademark and/or tradename. Mention of those trademarks in the present document does not constitute an endorsement by ETSI of products, services or organizations associated with those trademarks.

DECT™, PLUGTESTS™, UMTS™ and the ETSI logo are trademarks of ETSI registered for the benefit of its

Members. 3GPP™ and LTE™ are trademarks of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners. oneM2M™ logo is a trademark of ETSI registered for the benefit of its Members and of the oneM2M Partners. GSM® and the GSM logo are trademarks registered and owned by the GSM Association.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee Core Network and Interoperability Testing (INT).

The present document is part 2 of a multi-part deliverable. Full details of the entire series can be found in part 1 [2].

# Modal verbs terminology

In the present document "shall", "shall not", "should", "should not", "may", "need not", "will", "will not", "can" and "cannot" are to be interpreted as described in clause 3.2 of the ETSI Drafting Rules (Verbal forms for the expression of provisions).

"must" and "must not" are NOT allowed in ETSI deliverables except when used in direct citation.

# Scope

The present document provides the Test Suite Structure (TSS) and Test Purposes (TP) for the test specification for the NGAP protocol on the N2 interface as specified in ETSI TS 138 413 [1] in compliance with the relevant requirements and in accordance with the relevant guidance given in ISO/IEC 9646-7 [i.2] and ETSI ETS 300 406 [i.3].

# References

## Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at https://docbox.etsi.org/Reference.

NOTE: 	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

- ETSI TS 138 413 (V16.14.0): "5G; NG-RAN; NG Application Protocol (NGAP) (3GPP

TS 38.413 version 16.14.0 Release 16) ".

- ETSI TS 103 920-1: "Core Network and Interoperability Testing (INT); 5G NGAP Conformance Testing for the N2 interface; (3GPPTM Release 16); Part 1: Protocol Implementation Conformance Statement (PICS)".

- ETSI TS 123 502 (V16.15.0): "5G; Procedures for the 5G System (5GS) (3GPP TS 23.502 version 16.15.0 Release 16)".

## Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE: 	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1] 	ISO/IEC 9646-1: "Information technology -- Open Systems Interconnection -- Conformance testing methodology and framework -- Part 1: General concepts".

[i.2] 	ISO/IEC 9646-7: "Information technology -- Open Systems Interconnection -- Conformance testing methodology and framework -- Part 7: Implementation Conformance Statements".

[i.3] ETSI ETS 300 406: "Methods for testing and Specification (MTS); Protocol and profile conformance testing specifications; Standardization methodology".

# Definition of terms, symbols and abbreviations

## Terms

For the purposes of the present document, the terms given in ETSI TS 138 413 [1] and the following apply:

Abstract Test Method (ATM): Refer to ISO/IEC 9646-1 [i.1].

Abstract Test Suite (ATS): Refer to ISO/IEC 9646-1 [i.1].

Implementation Under Test (IUT): Refer to ISO/IEC 9646-1 [i.1].

Test Purpose (TP): Refer to ISO/IEC 9646-1 [i.1].

## Symbols

Void.

## Abbreviations

For the purposes of the present document, the abbreviations given in ETSI TS 138 413 [1] and the following apply:

TS 	Test System

TSS 	Test Suite Structure

# Test configurations

## Introduction

Test purposes of the present document address the 5G functional entities gNB and AMF that are accessible via the standardized N2 interface.

## Test configuration using the N2 interface

The N2 interface is located between the gNB and the AMF. Following configurations are simplified to highlight tested interface and involved entities. Overall network infrastructure is shown under clause 4.3.

Figure 1: Test configuration CF_AMF_N2

TS

IUT

N2

AMF

UE

gNB

Uu

Figure 1: Test configuration CF_AMF_N2

TS

IUT

N2

AMF

UE

gNB

Uu

TS

IUT

N2

AMF

source

gNB

target

gNB

N2

UE

Uu

TS

IUT

N2

AMF

source

gNB

target

gNB

N2

UE

Uu

Figure 2: Test configuration CF_AMF_2N2

IUT

TS

N2

AMF

gNB

UE

Uu

IUT

TS

N2

AMF

gNB

UE

Uu

Figure 3: Test configuration CF_GNB_N2

IUT

TS

N2

AMF

source

gNB

target

gNB

N2

UE

Uu

IUT

TS

N2

AMF

source

gNB

target

gNB

N2

UE

Uu

Figure 4: Test configuration CF_GNB_2N2

## Network infrastructure

Figure 5: Network architecture

# Test Suite Structure (TSS) and Test Purposes (TP)

## Test Suite Structure

### TP naming convention

TPs are numbered, starting at 01, within each group. Groups are organized according to the TSS.

Table 1: TP identifier naming convention scheme

|  | Identifier: <TP>_<iut>_<scope>_<nn> <tp> = Test Purpose: |  | fixed to "TP" |  |
| --- | --- | --- | --- | --- |
|  | <interface or protocol> |  | Interface or protocol: NGAP |  |
|  | <iut> = | type of IUT: | GNB or AMF |  |
|  | <scope> | = group | PDU | PDU Session Management Procedures |
|  |  |  | CMP | Context Management procedures |
|  |  |  | MMP | UE Mobility Management Procedures |
|  |  |  | PAG | Paging |
|  |  |  | NAS | Transport of NAS Messages Procedures |
|  |  |  | IMP | Interface Management Procedures |
|  |  |  | CTP | Configuration Transfer Procedures |
|  |  |  | WTP | Warning Message Transmission Procedures |
|  |  |  | NTP | NRPPa transport Procedures |
|  |  |  | TRP | Trace Procedures |
|  |  |  | LRP | Location Reporting Procedures |
|  |  |  | UBP | UE TNLA Binding Procedures |
|  |  |  | URP | UE Radio Capability Management Procedures |
|  |  |  | DRP | Data Usage Reporting Procedures |
|  |  |  | RIP | RIM Information Transfer Procedures |
|  | <nn> = | sequential number | (01 to 99) |  |

### Test strategy

As the base specification in ETSI TS 138 413 [1] contains no explicit requirements for testing, the TPs were generated as a result of an analysis of the base standard and the PICS specification ETSI TS 103 920-1 [2].

### TP structure

Each TP has been written in a manner which is consistent with all other TPs. The intention of this is to make the TPs more readable and checkable. A particular structure has been used which is illustrated in Table 2. Table 2 should be read in conjunction with any TP, i.e. use a TP as an example to facilitate the full comprehension of Table 2.

Table 2: Structure of a single TP

| TP part | Text | Example |
| --- | --- | --- |
| Header | <Identifier> | see Table 1 |
|  | <clause number in base ETSI TS 138 413 [1]> | clause 8.3.11.2 |
|  | <PICS reference> | A.3/3.1 |
| Summary | Short free text description of the test objective | Verify that the IUT can send a HANDOVER REQUIRED message containing mandatory IEs due to UE mobility management procedure. |
| Configuration | Test configuration as described in clause 4.2 | CFG_GNB_01 |
| Initial condition (optional) | Free text description of the condition that the IUT has reached before the test purpose applies. |  |
| Start point | Ensure that the IUT in the | Handover Preparation |
|  | <state> see ETSI TS 138 413 [1], clause 8.1 and/or further actions before stimulus if the action is sending/receiving see below for message structure | having sent a HANDOVER_REQUIRED |
| Stimulus | <trigger>, see below for message structure or <goal> | on receipt of a HANDOVER_COMMAND (see note 2) |
| Reaction | <action>. if the action is sending see below for message structure <next action>, etc. | sends, saves, does, etc. |
| Message structure | <message type> - containing a(n) <IE name> IE (see note 4) - indicating <coding of the field> and back to a) or b) (see note 3) | Message exchange, etc. (see note 2) |
| NOTE 1: Text in italics will not appear in TPs and text between <> is filled in for each TP and may differ from one TP to the next. NOTE 2: All messages are considered as "valid and compatible" unless otherwise specified in the test purpose. This includes the presence of all mandatory IEs as specified in ETSI TS 138 413 [1]. NOTE 3: An IE can be embedded into another IE. This is expressed by indentations, e.g. if Message1 contains IE1 and IE2 where IE1 has IE3 embedded this will be expressed like this: sends/receives Message 1 containing IE1 containing IE3 indicating ... containing IE2 indicating ... NOTE 4: IE value fields used for e.g. identification or address should be equal in the scope of TP if not stated otherwise. |  |  |

ETSI TS 103 920

## Test Purposes

### PICS references

All PICS items referred to in this clause are as specified in ETSI TS 103 920-1 [2] unless indicated otherwise by another numbered reference. PICS items are only meant for test selection, therefore only PICS items with status optional or conditional are explicitly mentioned.

### N2 interface

#### gNB Role

##### Test selection

The IUT takes the role of the gNB; PICS A.2/1.

##### PDU Session Management Procedures

| TP Id | TP_NGAP_GNB_PDU_01 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a PDU SESSION RESOURCE SETUP REQUEST containing mandatory IEs and answers with PDU SESSION RESOURCE SETUP RESPONSE for successfully established PDU session. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.1.2, 9.2.1.1 and 9.2.1.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/1_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PDU_SESSION_RESOURCE_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Setup_Request_List containing PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI, PDU_Session_Resource_Setup_Request_Transfer containing UL_NG_U_UP_TNL, PDU_SessionType, QoSFlowSetupRequestList containing QoSFlowSetupRequestItem containing QoSFlowIndicator, QoSFlowLevelQoSParameters from the AMF entity } then { the IUT sends a PDU_SESSION_RESOURCE_SETUP_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Setup_Response_List containing PDU_Session_Resource_Setup_Response_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Setup_Response_Transfer containing DL_QoS_Flow_per_TNL_Information to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_PDU_02 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node reports the establishment of the corresponding PDU session as failed if a PDU SESSION RESOURCE SETUP REQUEST containing several PDU Session ID IEs set to the same value. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.1.4, 9.2.1.1 and 9.2.1.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/1_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PDU_SESSION_RESOURCE_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Setup_Request_List containing PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI, PDU_Session_Resource_Setup_Request_Transfer containing UL_NG_U_UP_TNL, PDU_SessionType, QoSFlowSetupRequestList containing QoSFlowSetupRequestItem containing QoSFlowIndicator, QoSFlowLevelQoSParameters, PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI, PDU_Session_Resource_Setup_Request_Transfer containing UL_NG_U_UP_TNL, PDU_SessionType, QoSFlowSetupRequestList containing QoSFlowSetupRequestItem containing QoSFlowIndicator, QoSFlowLevelQoSParameters from the AMF entity } then { the IUT sends a PDU_SESSION_RESOURCE_SETUP_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Failed_to_Setup_List containing DU_Session_Resource_Failed_to_Setup_Item containing PDU_SessionId, PDU_Session_Resource_Setup_Unsuccessful_Transfer containing Cause indicating value MultiplePDUSessionIDInstances to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_PDU_03 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node reports the establishment of the new PDU session as failed if a PDU SESSION RESOURCE SETUP REQUEST containing a PDU Session ID IE set to value that identifies an active PDU Session. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.1.4, 9.2.1.1 and 9.2.1.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/1_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedPDUsessionWithSameId } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PDU_SESSION_RESOURCE_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Setup_Request_List containing PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI, PDU_Session_Resource_Setup_Request_Transfer containing UL_NG_U_UP_TNL, PDU_SessionType, QoSFlowSetupRequestList containing QoSFlowSetupRequestItem containing QoSFlowIndicator, QoSFlowLevelQoSParameters from the AMF entity } then { the IUT sends a PDU_SESSION_RESOURCE_SETUP_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Failed_to_Setup_List containing DU_Session_Resource_Failed_to_Setup_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Setup_Unsuccessful_Transfer containing Cause indicating value MultiplePDUSessionIDInstances to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_PDU_04 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node reports the establishment of the corresponding PDU session as failed if a PDU SESSION RESOURCE SETUP REQUEST containing a QoS Flow Setup Request List IE in the PDU Session Resource Setup Request Transfer IE including at least one Non-GBR QoS flow but the PDU Session Aggregate Maximum Bit Rate IE is not present. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.1.4, 9.2.1.1 and 9.2.1.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/1_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PDU_SESSION_RESOURCE_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Setup_Request_List containing PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI, PDU_Session_Resource_Setup_Request_Transfer containing UL_NG_U_UP_TNL, PDU_SessionType, QoSFlowSetupRequestList containing QoSFlowSetupRequestItem containing QoSFlowIndicator indicating value Non_GBR_QoS, QoSFlowLevelQoSParameters, |  |
| not UEAggregateMaximumBitRate from the AMF entity } then { the IUT sends a PDU_SESSION_RESOURCE_SETUP_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Failed_to_Setup_List containing DU_Session_Resource_Failed_to_Setup_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Setup_Unsuccessful_Transfer containing Cause indicating value InvalidQoSCombination to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_PDU_05 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a PDU SESSION RESOURCE RELEASE COMMAND containing mandatory IEs and answers with PDU SESSION RESOURCE RELEASE RESPONSE to release PDU session. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.2.2, 9.2.1.3 and 9.2.1.4 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/1_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PDU_SESSION_RESOURCE_RELEASE_COMMAND containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_To_Release_List containing PDU_Session_Resource_To_Release_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI, PDU_Session_Resource_Release_Command_Transfer containing Cause indicating value PX_Cause //normal_release from the AMF entity } then { the IUT sends a PDU_SESSION_RESOURCE_RELEASE_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Released_List containing PDU_Session_Resource_Released_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Release_Response_Transfer to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_PDU_06 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a PDU SESSION RESOURCE RELEASE COMMAND containing multiple PDU Session ID IEs set to the same value and ignore the duplication. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.2.2, 9.2.1.3 and 9.2.1.4 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/1_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PDU_SESSION_RESOURCE_RELEASE_COMMAND containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_To_Release_List containing PDU_Session_Resource_To_Release_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI, PDU_Session_Resource_Release_Command_Transfer containing Cause indicating value PX_Cause //normal_release , PDU_Session_Resource_To_Release_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI, PDU_Session_Resource_Release_Command_Transfer containing Cause indicating value PX_Cause //normal_release from the AMF entity } then { the IUT sends a PDU_SESSION_RESOURCE_RELEASE_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Released_List containing PDU_Session_Resource_Released_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Release_Response_Transfer to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_PDU_07 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a PDU SESSION RESOURCE MODIFY REQUEST containing mandatory IEs and answers with PDU SESSION RESOURCE MODIFY RESPONSE for successfully modified PDU session. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.3.2, 9.2.1.5 and 9.2.1.6 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/1_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PDU_SESSION_RESOURCE_MODIFY_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Modify_Request_List containing PDU_Session_Resource_Modify_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Modify_Request_Transfer containing QoSFlowAddorModifyRequestList containing QoSFlowAddorModifyRequestItem containing QoSFlowIdentifier from the AMF entity } then { the IUT sends a PDU_SESSION_RESOURCE_MODIFY_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Modify_Response_List containing PDU_Session_Resource_Modify_Response_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Modify_Response_Transfer containing QoSFlowAddorModifyResponsetList containing QoSFlowAddorModifyResponsetItem containing QoSFlowIdentifier to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_PDU_08 |
| --- | --- |
| Test Objective | Verify that the IUT can send a PDU_SESSION_RESOURCE_NOTIFY that already established QoS flow or PDU session for given UE are released. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.4.2 and 9.2.1.7 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/1_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a PDU session resource notify procedure } then { the IUT sends a PDU_SESSION_RESOURCE_NOTIFY containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Notify_List containing PDU_Session_Resource_Notify_Item to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_PDU_09 |
| --- | --- |
| Test Objective | Verify that the IUT can send a PDU_SESSION_RESOURCE_MODIFY_INDICATION to request modification of established PDU session. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.5.2 and 9.2.1.8 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/1_5 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a PDU session resource modify indicationy procedure } then { the IUT sends a PDU_SESSION_RESOURCE_MODIFY_INDICATION containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Modify_Indication_List containing PDU_Session_Resource_Modify_Indication_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Modify_Indication_Transfer containing DLQoSFlowperTNLInformation to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_PDU_10 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node reports the modification of the corresponding PDU session as failed if a PDU SESSION RESOURCE MODIFY REQUEST contains several PDU Session ID IEs set to the same value. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.3.4, 9.2.1.5 and 9.2.1.6 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/1_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedPDUsession } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PDU_SESSION_RESOURCE_MODIFY_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Modify_Request_List containing PDU_Session_Resource_Modify_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Modify_Request_Transfer containing QoSFlowAddorModifyRequestList containing QoSFlowAddorModifyRequestItem containing QoSFlowIdentifier, PDU_Session_Resource_Modify_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Modify_Request_Transfer containing QoSFlowAddorModifyRequestList containing QoSFlowAddorModifyRequestItem containing QoSFlowIdentifier from the AMF entity } then { the IUT sends a PDU_SESSION_RESOURCE_MODIFY_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Failed_to_Modify_Response_List containing PDU_Session_Resource_Failed_to_Modify_Response_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Modify_Unsuccessful_Transfer containing Cause indicating value MultiplePDUSessionIDInstances to the AMF entity } } |  |

##### UE Context Management Procedures

| TP Id | TP_NGAP_GNB_CMP_01 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes an INITIAL CONTEXT SETUP REQUEST containing optional PDU Session Resource Setup Request List field mandatory IEs and answers with INITIAL CONTEXT SETUP RESPONSE with successfully established UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.1.2, 9.2.2.1 and 9.2.2.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an INITIAL_CONTEXT_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, PDU_Session_Resource_Setup_Request_List containing PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, PDU_Session_Resource_Setup_Request_Transfer, Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, |  |

Security_Key from the AMF entity } then { the IUT sends an INITIAL_CONTEXT_SETUP_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Setup_Response_List containing PDU_SessionId, PDU_Session_Resource_Setup_Response_Transfer to the AMF entity } }

| TP Id | TP_NGAP_GNB_CMP_02 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes an INITIAL CONTEXT SETUP REQUEST containing mandatory IEs and answers with INITIAL CONTEXT SETUP RESPONSE with successfully established UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.1.2, 9.2.2.1 and 9.2.2.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an INITIAL_CONTEXT_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, Security_Key from the AMF entity } then { the IUT sends an INITIAL_CONTEXT_SETUP_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_CMP_03 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes an INITIAL CONTEXT SETUP REQUEST with optional field Trace Activation and answers with an INITIAL CONTEXT SETUP RESPONSE with successfully established UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.1.2, 9.2.2.1 and 9.2.2.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an INITIAL_CONTEXT_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, PDU_Session_Resource_Setup_Request_List containing PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, PDU_Session_Resource_Setup_Request_Transfer, Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms , Security_Key, Trace_Activation containing NG_RAN_Trace_ID, Interfaces_to_Trace, Trace_Depth, Trace_Collection_Entity_IP_Address from the AMF entity } then { the IUT sends an INITIAL_CONTEXT_SETUP_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Setup_Response_List containing PDU_SessionId, PDU_Session_Resource_Setup_Response_Transfer to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_CMP_04 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes an INITIAL CONTEXT SETUP REQUEST with optional field Mobility Restriction List and answers with INITIAL CONTEXT SETUP RESPONSE with successfully established UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.1.2, 9.2.2.1 and 9.2.2.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an INITIAL_CONTEXT_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, PDU_Session_Resource_Setup_Request_List containing PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, PDU_Session_Resource_Setup_Request_Transfer, |  |
| Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, Security_Key, Mobility_Restriction_List containing Serving_PLMN from the AMF entity } then { the IUT sends an INITIAL_CONTEXT_SETUP_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Setup_Response_List containing PDU_SessionId, PDU_Session_Resource_Setup_Response_Transfer to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_CMP_05 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes an INITIAL CONTEXT SETUP REQUEST with different optional fields and answers with INITIAL CONTEXT SETUP RESPONSE with successfully established UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.1.2, 9.2.2.1 and 9.2.2.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an INITIAL_CONTEXT_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, PDU_Session_Resource_Setup_Request_List containing PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, PDU_Session_Resource_Setup_Request_Transfer, Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, Security_Key, UE_Radio_Capability, Index_to_RAT_Frequency_Selection_Priority, Masked_IMEISV, NAS_PDU, Emergency_Fallback_Indicator, RRC_Inactive_Transition_Report_Request, Redirection_for_Voice_EPS_Fallback, Location_Reporting_Request_Type containing Event_Type, Report_Area, |  |

2 V1.1.1 (2024-10)

SRVCC_Operation_Possible, IAB_Authorized, Enhanced_Coverage_Restriction, Extended_Connected_Time, UE Differentiation Information containing Periodic_Communication_Indicator, Periodic_Time, NR_UE_Sidelink_Aggregate_Maximum_Bit_Rate, LTE_UE_Sidelink_Aggregate_Maximum_Bit_Rate, UE_Radio_Capability_ID from the AMF entity } then { the IUT sends an INITIAL_CONTEXT_SETUP_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Setup_Response_List containing PDU_SessionId, PDU_Session_Resource_Setup_Response_Transfer to the AMF entity } }

| TP Id | TP_NGAP_GNB_CMP_06 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes an INITIAL CONTEXT SETUP REQUEST containing mandatory IEs and answers with INITIAL CONTEXT SETUP RESPONSE with failed PDU session. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.1.2, 9.2.2.1 and 9.2.2.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedContextInproperly } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an INITIAL_CONTEXT_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, PDU_Session_Resource_Setup_Request_List containing PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, PDU_Session_Resource_Setup_Request_Transfer, Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, Security_Key from the AMF entity } then { the IUT sends an INITIAL_CONTEXT_SETUP_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Failed_to_Setup_List containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Setup_Unsuccessful_Transfer to the AMF entity } } |  |
| TP Id | TP_NGAP_GNB_CMP_07 |
| Test Objective | Verify that the NG-RAN node successfully requests the AMF to release the UE-associated logical NG-connection. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.2.2 and 9.2.2.4 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a UE context release request } then { the IUT sends a UE_CONTEXT_RELEASE_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Cause to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_CMP_09 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a UE CONTEXT RELEASE COMMAND where only the AMF UE NGAP ID IE is available and answers with UE CONTEXT RELEASE COMPLETE with successfully release UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.3, 9.2.2.6 and 9.2.2.5 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |

| TP Id | TP_NGAP_GNB_CMP_08 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a UE CONTEXT RELEASE COMMAND that contains both the AMF UE NGAP ID IE and the RAN UE NGAP ID IE and answers with UE CONTEXT RELEASE COMPLETE with successfully release UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.3, 9.2.2.6 and 9.2.2.5 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a UE_CONTEXT_RELEASE_COMMAND containing CHOICE_UE_NGAP_IDs containing UE_NGAP_ID_pair containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID from the AMF entity } then { the IUT sends a UE_CONTEXT_RELEASE_COMPLETE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the AMF entity } } |  |

Expected Behaviour

ensure that { when { the IUT receives a UE_CONTEXT_RELEASE_COMMAND containing CHOICE_UE_NGAP_IDs containing AMF_UE_NGAP_ID containing AMF_UE_NGAP_ID from the AMF entity } then { the IUT sends a UE_CONTEXT_RELEASE_COMPLETE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the AMF entity } }

| TP Id | TP_NGAP_GNB_CMP_10 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a UE CONTEXT MODIFICATION REQUEST containing mandatory IEs and answers with UE CONTEXT MODIFICATION FAILURE because the UE is no longer available. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.4.3, 9.2.2.7 and 9.2.2.9 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_4 |
| Initial Conditions |  |
| with { the UE isNoLongerAvailable } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a UE_CONTEXT_MODIFICATION_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID from the AMF entity } then { the IUT sends a UE_CONTEXT_MODIFICATION_FAILURE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Cause to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_CMP_11 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a UE CONTEXT MODIFICATION REQUEST containing mandatory IEs after the preparation of a handover and answers with UE CONTEXT MODIFICATION RESPONSE with successfully modify UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.4.2, 9.2.2.7 and 9.2.2.8 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext and the GNB alreadyPreparedHandover } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a UE_CONTEXT_MODIFICATION_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID from the AMF entity } then { the IUT sends a UE_CONTEXT_MODIFICATION_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the AMF entity |  |

}

}

| TP Id | TP_NGAP_GNB_CMP_12 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a UE CONTEXT MODIFICATION REQUEST containing optional fields Security Key and Security Capabilities and answers with UE CONTEXT MODIFICATION RESPONSE with successfully modify UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.4.2, 9.2.2.7 and 9.2.2.8 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a UE_CONTEXT_MODIFICATION_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Security_Key, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms from the AMF entity } then { the IUT sends a UE_CONTEXT_MODIFICATION_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_CMP_13 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a UE CONTEXT MODIFICATION REQUEST containing different optional fields and answers with UE CONTEXT MODIFICATION RESPONSE with successfully modify UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.4.2, 9.2.2.7 and 9.2.2.8 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a UE_CONTEXT_MODIFICATION_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, RAN_Paging_Priority, Index_to_RAT_Frequency_Selection_Priority, UE_Aggregate_Maximum_Bit_Rate containing UE_Aggregate_Maximum_Bit_Rate_Downlink, UE_Aggregate_Maximum_Bit_Rate_Uplink, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, Emergency_Fallback_Indicator, New_GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, IAB_Authorized, |  |
| PC5_QoS_Parameters containing PC5_QoS_Flow_List containing PC5_QoS_Flow_Item containing PQI, UE_Radio_Capability_ID, RG_Level_Wireline_Access_Characteristics from the AMF entity } then { the IUT sends a UE_CONTEXT_MODIFICATION_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_CMP_14 |
| --- | --- |
| Test Objective | Verify that the IUT can send an RRC INACTIVE TRANSITION REPORT that contains mandatory IEs |
| Reference | ETSI TS 138 413 [1], clauses 8.3.5 and 9.2.2.10 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/2_5 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE isTransitioningTo the RRC_INACTIVE } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate an RRC inactive transition report } then { the IUT sends an RRC_INACTIVE_TRANSITION_REPORT containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, RRC_State, User_Location_Information to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_CMP_15 |
| --- | --- |
| Test Objective | Verify that the IUT can send a RAN CP Relocation Indication that contains mandatory IEs. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.8.2 and 9.2.2.13 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1_2 and PICS_A3/2_8 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a RAN CP relocation indication procedure } then { the IUT sends a RAN_CP_RELOCATION_INDICATION containing RAN_UE_NGAP_ID, 5G_S_TMSI containing AMF_Set_ID, AMF_Pointer, 5G_TMSI, E_ULTRA_CGI containing PLMN_Identity, E_UTRA_Cell_Identity, TAI containing PLMN_Identity, TAC, UL_CP_Security_Information containing UL_NAS_MAC, |  |

UL_NAS_Count                         to the AMF entity

}

}

| TP Id | TP_NGAP_GNB_CMP_16 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UE CONTEXT SUSPEND REQUEST containing mandatory IEs |
| Reference | ETSI TS 138 413 [1], clauses 8.3.11.2 and 9.2.2.16 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1_2 and PICS_A3/2_11 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a UE context suspend request } then { the IUT sends a UE_CONTEXT_SUSPEND_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_MMP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a HANDOVER REQUIRED message containing mandatory IEs. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.1.2 and 9.2.3.1 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |

| TP Id | TP_NGAP_GNB_CMP_17 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UE CONTEXT RESUME REQUEST containing mandatory IEs |
| Reference | ETSI TS 138 413 [1], clauses 8.3.12.2 and 9.2.2.19 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1_2 and PICS_A3/2_12 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasSuspendedContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a UE context resume request } then { the IUT sends a UE_CONTEXT_RESUME_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, RRC_Resume_Cause to the AMF entity } } |  |

##### UE Mobility Management Procedures

Expected Behaviour

ensure that { when { the IUT indicate the initiation "of a Handover Required procedure" } then { the IUT sends a HANDOVER_REQUIRED containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Handover_Typ, Cause, Target_ID, PDU_Session_Resource_List containing PDU_Session_Resource_Item containing PDU_SessionId indicating value PX_PDU_ID, Handover_Required_Transfer, Source_to_Target_Transparent_Container to the AMF entity } }

| TP Id | TP_NGAP_GNB_MMP_02 |
| --- | --- |
| Test Objective | Verify that the GNB node successfully processes a HANDOVER REQUEST message that contains mandatory IEs and answers with HANDOVER REQUEST ACKNOWLEDGE to acknowledge the handover. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.2.2, 9.2.3.4, 9.2.3.5, 9.3.4.1 and 9.3.4.11 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a HANDOVER_REQUEST containing AMF_UE_NGAP_ID, Handover_Typ, Cause, UE_Aggregate_Maximum_Bit_Rate, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, Security_Context containing Next_Hop_Chaining_Count, Next_Hop_NH, PDU_Session_Resource_Setup_List containing PDU_Session_Resource_Setup_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, Handover_Request_Transfer containing DL_NG_U_UP_TNL_Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, PDU_Session_Type, QoS_Flow_Setup_Request_List containing QoS_Flow_Setup_Request_Item containing QoS_Flow_Identifier, QoS_Flow_Level_QoS_Parameter, Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, Source_to_Target_Transparent_Container, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer from the AMF entity |  |

2 V1.1.1 (2024

} then { the IUT sends a HANDOVER_REQUEST_ACKNOWLEDGE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Admitted_List containing PDU_Session_Resource_Admitted_Item containing PDU_SessionId indicating value PX_PDU_ID, Handover_Request_Acknowledge_Transfer containing DL_NG_U_UP_TNL_Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, QoS_Flow_Setup_Response_List containing QoS_Flow_Item_with_Data_Forwarding containing QoS_Flow_Identifier, Target_to_Source_Transparent_Container to the AMF entity } }

| TP Id | TP_NGAP_GNB_MMP_03 |
| --- | --- |
| Test Objective | Verify that the GNB node successfully processes a HANDOVER REQUEST message that contains mandatory IEs and answers with HANDOVER REQUEST FAILURE because the UE is not longer available. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.2.3, 9.2.3.4, 9.2.3.6 and 9.3.4.1 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_2 |
| Initial Conditions |  |
| with { the UE isNoLongerAvailable } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a HANDOVER_REQUEST containing AMF_UE_NGAP_ID, Handover_Typ, Cause, UE_Aggregate_Maximum_Bit_Rate containing UE_Aggregate_Maximum_Bit_Rate_Downlink, UE_Aggregate_Maximum_Bit_Rate_Uplink, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, Security_Context containing Next_Hop_Chaining_Count, Next_Hop_NH, PDU_Session_Resource_Setup_List containing PDU_Session_Resource_Setup_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, Handover_Request_Transfer containing DL_NG_U_UP_TNL_Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, PDU_Session_Type, QoS_Flow_Setup_Request_List containing QoS_Flow_Setup_Request_Item containing QoS_Flow_Identifier, QoS_Flow_Level_QoS_Parameter, Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, Source_to_Target_Transparent_Container, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, |  |
| AMF_Pointer from the AMF entity } then { the IUT sends a HANDOVER_FAILURE containing AMF_UE_NGAP_ID, Cause to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_MMP_04 |
| --- | --- |
| Test Objective | Verify that the GNB node successfully processes a HANDOVER REQUEST message that contains optional IEs and answers with HANDOVER REQUEST ACKNOWLEDGE to acknowledge the handover. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.2.2, 9.2.3.4, 9.2.3.5, 9.3.4.1 and 9.3.4.11 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a HANDOVER_REQUEST containing AMF_UE_NGAP_ID, Handover_Typ, Cause, UE_Aggregate_Maximum_Bit_Rate containing UE_Aggregate_Maximum_Bit_Rate_Downli, Core_Network_Assistance_Information_for_RRC_INACTIVE containing UE_Identity_Index_Value containing CHOICE_UE_Identity_Index_Value containing Index_Length_10 containing Index_Length_10, Periodic_Registration_Update_Timer, TAI_List_for_RRC_Inactive containing TAI_List_for_RRC_Inactive_Item containing TAI containing PLMN_Identity, TAC, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, Security_Context containing Next_Hop_Chaining_Count, Next_Hop_NH, New_Security_Context_Indicator, NASC containing NAS_PDU, PDU_Session_Resource_Setup_List containing PDU_Session_Resource_Setup_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, Handover_Request_Transfer containing DL_NG_U_UP_TNL_Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, PDU_Session_Type, QoS_Flow_Setup_Request_List containing QoS_Flow_Setup_Request_Item containing QoS_Flow_Identifier, QoS_Flow_Level_QoS_Parameter, Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, Trace_Activiation containing NG_RAN_TRACE_ID, |  |
| Interfaces_to_Trace, Trace_Depth, Trace_Collection_Entity_IP_Address containing Transport_Layer_Address, Source_to_Target_Transparent_Container, RRC_Inactive_Transition_Report_Request, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, Redirection_for_Voice_EPS_Fallback, CN_Assisted_RAN_Parameters_Tuning, SRVCC_Operation_Possible, Enhanced_Coverage_Restriction, UE_Differentiation_Information, NR_V2X_Services_Authorized, LTE_V2X_Services_Authorized, NR_UE_Sidelink_Aggregate_Maximum_Bit_Rate, LTE_UE_Sidelink_Aggregate_Maximum_Bit_Rate, PC5_QoS_Parameters containing PC5_QoS_Flow_List containing PC5_QoS_Flow_Item containing PQI, PC5_Flow_Bit_Rates containing Guaranteed_Flow_Bit_Rate, Maximum_Flow_Bit_Rate, CE_mode_B_Restricted, UE_User_Plane_CIoT_Support_Indicator, Management_Based_MDT_PLMN_List containing PLMN_Identity, UE_Radio_Capability_ID, Extended_Connected_Time from the AMF entity } then { the IUT sends a HANDOVER_REQUEST_ACKNOWLEDGE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Admitted_List containing PDU_Session_Resource_Admitted_Item containing PDU_SessionId indicating value PX_PDU_ID, Handover_Request_Acknowledge_Transfer containing DL_NG_U_UP_TNL_Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, QoS_Flow_Setup_Response_List containing QoS_Flow_Item_with_Data_Forwarding containing QoS_Flow_Identifier, Target_to_Source_Transparent_Container to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_MMP_05 |
| --- | --- |
| Test Objective | Verify that the GNB node successfully processes a HANDOVER REQUEST message that contains optional IEs within the Handover Request Transfer IEs and answers with correct HANDOVER REQUEST ACKNOWLEDGE. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.2.2, 9.2.3.4, 9.2.3.5, 9.3.4.1 and 9.3.4.11 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a HANDOVER_REQUEST containing AMF_UE_NGAP_ID, Handover_Typ, Cause, UE_Aggregate_Maximum_Bit_Rate containing UE_Aggregate_Maximum_Bit_Rate_Downlink, |  |
| UE_Aggregate_Maximum_Bit_Rate_Uplink, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, Security_Context containing Next_Hop_Chaining_Count, Next_Hop_NH, PDU_Session_Resource_Setup_List containing PDU_Session_Resource_Setup_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, Handover_Request_Transfer containing DL_NG_U_UP_TNL_Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, Data_Forwarding_Not_Possible indicating value "data forwarding not possible", Security_Indication containing Integrity_Protection_Indication indicating value preferred, Confidentiality_Protection_Indication indicating value preferred, Maximum_Integrity_Protected_Data_Rate_Uplink indicating value max_UE_rate, PDU_Session_Type, QoS_Flow_Setup_Request_List containing QoS_Flow_Setup_Request_Item containing QoS_Flow_Identifier, QoS_Flow_Level_QoS_Parameter, Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, Source_to_Target_Transparent_Container, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer from the AMF entity } then { the IUT sends a HANDOVER_REQUEST_ACKNOWLEDGE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Admitted_List containing PDU_Session_Resource_Admitted_Item containing PDU_SessionId indicating value PX_PDU_ID, Handover_Request_Acknowledge_Transfer containing DL_NG_U_UP_TNL_Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, DL Forwarding UP TNL Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, Security_Result containing Integrity_Protection_Result indicating value performed, Confidentiality_Protection_Result indicating value performed, QoS_Flow_Setup_Response_List containing QoS_Flow_Item_with_Data_Forwarding containing QoS_Flow_Identifier, Target_to_Source_Transparent_Container to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_MMP_06 |
| --- | --- |
| Test Objective | Verify that the GNB node successfully processes a HANDOVER REQUEST message for an intrasystem handover and answers with HANDOVER REQUEST ACKNOWLEDGE to acknowledge the handover. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.2.2, 9.2.3.4, 9.2.3.5, 9.3.4.1 and 9.3.4.11 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a HANDOVER_REQUEST containing AMF_UE_NGAP_ID, Handover_Typ, Cause, UE_Aggregate_Maximum_Bit_Rate containing UE_Aggregate_Maximum_Bit_Rate_Downlink, UE_Aggregate_Maximum_Bit_Rate_Uplink, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, Security_Context containing Next_Hop_Chaining_Count, Next_Hop_NH, PDU_Session_Resource_Setup_List containing PDU_Session_Resource_Setup_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, Handover_Request_Transfer containing DL_NG_U_UP_TNL_Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, PDU_Session_Type, QoS_Flow_Setup_Request_List containing QoS_Flow_Setup_Request_Item containing QoS_Flow_Identifier, QoS_Flow_Level_QoS_Parameter, Direct_Forwarding_Path_Availability indicating value "direct path available", Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, Source_to_Target_Transparent_Container containing RRC_Container, PDU_Session_Resource_Information_List containing PDU_Session_Resource_Information_Item containing PDU_Session_ID indicating value PX_PDU_ID, QoS_Flow_Information_List containing QoS_Flow_Information_Item containing QoS_Flow_Identifier, DL_Forwarding indicating value "DL forwarding proposed", UL_Forwarding indicating value "UL forwarding proposed", Target_Cell_ID containing CHOICE_NG_RAN_CGI containing NR containing NR_CGI, UE_History_Information containing CHOICE_UE_History_Information_from_UE containing NR containing NR_Mobility_History_Report, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer from the AMF entity |  |
| } then { the IUT sends a HANDOVER_REQUEST_ACKNOWLEDGE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Admitted_List containing PDU_Session_Resource_Admitted_Item containing PDU_SessionId indicating value PX_PDU_ID, Handover_Request_Acknowledge_Transfer containing DL_NG_U_UP_TNL_Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, DL_Forwarding_UP_TNL_Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, QoS_Flow_Setup_Response_List containing QoS_Flow_Item_with_Data_Forwarding containing QoS_Flow_Identifier, UL_Forwarding_UP_TNL_Information containing CHOICE_UP_Transport_Layer_Information containing GTP_tunnel containing Endpoint_IP_Address, Target_to_Source_Transparent_Container to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_MMP_07 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully sends a HANDOVER NOTIFICATION message that contains mandatory IEs to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.3.2 and 9.2.3.7 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of a Handover Notify procedure" } then { the IUT sends a HANDOVER_NOTIFY containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, User_Location_Information to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_MMP_08 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully sends a PATH SWITCH REQUEST message that contains mandatory IEs to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.4.2 and 9.2.3.8 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of a Path Switch Request procedure" } then { the IUT sends a PATH_SWITCH_REQUEST containing RAN_UE_NGAP_ID, Source_AMF_UE_NGAP_ID, User_Location_Information, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, PDU_Session_Resources_to_be_Switched_in_Downlink_List containing PDU_Session_Resources_to_be_Switched_in_Downlink_Item containing PDU_SessionId indicating value PX_PDU_ID, Path_Switch_Request_Transfer to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_MMP_09 |
| --- | --- |
| Test Objective | Verify that the GNB node successfully sends a HANDOVER CANCEL message that contains mandatory IEs to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.5.2 and 9.2.3.11 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_5 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the GNB alreadyPreparedHandover } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of a Handover Cancel procedure" } then { the IUT sends a HANDOVER_CANCEL containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Cause to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_MMP_10 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully send a UPLINK RAN STATUS TRANSFER message to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.6.2 and 9.2.3.13 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_6 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the GNB alreadyPreparedHandover } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of a UE Uplink RAN Status Transfer procedure" } then { the IUT sends a UPLINK_RAN_STATUS_TRANSFER containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, RAN_Status_Transfer_Transparent_Container to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_MMP_11 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully sends a UPLINK RAN EARLY STATUS TRANSFER message to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.9.2 and 9.2.3.16 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_9 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the GNB alreadyPreparedHandover } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of an Uplink RAN Early Status Transfer" } then { the IUT sends a UPLINK_RAN_EARLY_STATUS_TRANSFER containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Early_Status_Transfer_Transparent_Container to the AMF entity } } |  |

##### Paging Procedures

Void.

##### Transport of NAS Messages Procedures

| TP Id | TP_NGAP_GNB_NAS_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send an INITIAL UE MESSAGE to indicate the initiation of a NAS Transport procedure |
| Reference | ETSI TS 138 413 [1], clauses 8.6.1.2 and 9.2.5.1 ETSI TS 123 502 [3], clause 4.24.1 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/5_1 |
| Initial Conditions |  |
| with { the UE isCMIDLE and the UE hasEstablishedRRCConnection } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of a NAS Transport procedure" } then { the IUT sends an INITIAL_UE_MESSAGE containing RAN_UE_NGAP_ID, NAS_PDU, User_Location_Information, RRC_Establishmnet_Cause to the AMF entity |  |

}

}

| TP Id | TP_NGAP_GNB_NAS_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send an UPLINK NAS TRANSPORT message to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.6.3.2 and 9.2.5.3 ETSI TS 123 502 [3], clause 4.24.1 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/5_3 |
| Initial Conditions |  |
| with { the UE isCMCONNECTED in the GNB entity } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an RRC_UL_MESSAGE from the UE entity } then { the IUT sends an UPLINK_NAS_TRANSPORT containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, NAS_PDU, User_Location_Information to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_NAS_03 |
| --- | --- |
| Test Objective | Verify that the IUT can send a NAS NON DELIVERY INDICATION message to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.6.4.2 and 9.2.5.4 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/5_4 |
| Initial Conditions |  |
| with { the UE isNoLongerAvailable to the GNB entity } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of a NAS Non Delivery Indication procedure" } then { the IUT sends a NAS_NON_DELIVERY_INDICATION containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, NAS_PDU, Cause to the AMF entity } } |  |

##### Interface Management Procedures

| TP Id | TP_NGAP_GNB_IMP_01 |
| --- | --- |
| Test Objective | Verify that the GNB node successfully sends an NG SETUP REQUEST message to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.1.2 and 9.2.6.1 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/6_1 |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of an NG SETUP REQUEST" } then { the IUT sends an NG_SETUP_REQUEST containing Global_RAN_Node_ID, Supported_TA_List containing Supported_TA_Item containing TAC, Broadcast_PLMN_List containing Broadcast_PLMN_Item containing PLMN_Identity, TAI_Slice_Support_List containing S_NSSAI containing SST, Default_Paging_DRX to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_IMP_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send a RAN CONFIGURATION UPDATE message to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.2.2 and 9.2.6.4 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/2 and PICS_A3/6_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of a RAN CONFIGURATION UPDATE procedure" } then { the IUT sends a RAN_CONFIGURATION_UPDATE containing Supported_TA_List containing Supported_TA_Item containing TAC, Broadcast_PLMN_List containing Broadcast_PLMN_Item containing PLMN_Identity, TAI_Slice_Support_List containing S_NSSAI containing SST to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_IMP_03 |
| --- | --- |
| Test Objective | Verify that the GNB node successfully processes an AMF CONFIGURATION UPDATE message with AMF CONFIGURATION UPDATE ACKNOWLEDGE to acknowledge the update. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.3.2, 9.2.6.7 and 9.2.6.8 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/6_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE isCMIDLE and the UE hasReceivedPAGINGMessage } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an AMF_CONFIGURATION_UPDATE containing Served_GUAMI_List containing Served_GUAMI_Item containing GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, PLMN_Support_List containing PLMN_Support_Item containing PLMN_Identity, Slice_Support_List containing S_NSSAI containing SST, AMF_TNL_Association_to_Add_List containing AMF_TNL_Association_to_Add_item containing AMF_TNL_Association_Address, TNL_Address_Weight_Factor, AMF_TNL_Association_to_Update_List containing AMF_TNL_Association_to_Update_Item containing AMF_TNL_Association_Address from the AMF entity } then { the IUT sends an AMF_CONFIGURATION_UPDATE_ACKNOWLEDGE containing AMF_TNL_Association_Setup_List containing AMF_TNL_Association_Setup_Item containing AMF_TNL_Association_Address to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_IMP_04 |
| --- | --- |
| Test Objective | Verify that the GNB node can send an AMF CONFIGURATION UPDATE FAILURE. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.3.2, 9.2.6.7 and 9.2.6.9 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/6_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE isCMIDLE and the UE hasReceivedPAGINGMessage } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an AMF_CONFIGURATION_UPDATE containing Served_GUAMI_List containing Served_GUAMI_Item containing GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, PLMN_Support_List containing PLMN_Support_Item containing PLMN_Identity, Slice_Support_List containing S_NSSAI containing SST, AMF_TNL_Association_to_Add_List containing AMF_TNL_Association_to_Add_item containing AMF_TNL_Association_Address, TNL_Address_Weight_Factor, AMF_TNL_Association_to_Update_List containing AMF_TNL_Association_to_Update_Item containing AMF_TNL_Association_Address from the AMF entity } then { the IUT sends an AMF_CONFIGURATION_UPDATE_FAILURE containing Cause |  |

to the AMF entity

}

}

| TP Id | TP_NGAP_GNB_IMP_05 |
| --- | --- |
| Test Objective | Verify that the GNB node successfully processes an NG RESET message that contains mandatory IEs and answers with NG RESET ACKNOWLEDGE to acknowledge the reset. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.4.2.1, 9.2.6.11 and 9.2.6.12 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/6_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE isRequestedToDetachfromNetwork } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an NG_RESET containing Cause, CHOICE_Reset_Type containing NG_interface containing Reset_All from the AMF entity } then { the IUT sends an NG_RESET_ACKNOWLEDGE to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_IMP_06 |
| --- | --- |
| Test Objective | Verify that the GNB can send an ERROR INDICATION to the AMF when an error occurs. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.5.2 and 9.2.6.13 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/6_5 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate an ERROR INDICATION } then { the IUT sends an ERROR_INDICATION to the AMF entity } } |  |

##### Configuration Transfer Procedure

| TP Id | TP_NGAP_GNB_CTP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send an Uplink RAN Configuration Transfer message to transfer RAN configuration information to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.8.1 and 9.2.7.1 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/7_1 |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of an Uplink RAN Configuration Transfer" } then { the IUT sends an UPLINK_RAN_CONFIGURATION_TRANSFER to the AMF entity } } |  |

##### Warning Message Transmission Procedures

| TP Id | TP_NGAP_GNB_WTP_01 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a WRITE-REPLACE WARNING REQUEST containing mandatory IEs and answers with WRITE-REPLACE WARNING RESPONSE to start broadcasting of warning messages. |
| Reference | ETSI TS 138 413 [1], clauses 8.9.1.2 and 9.2.8.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/8_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a WRITE_REPLACE_WARNING_REQUEST containing Message_Identifier, Serial_Number, Repetition_Period, Number_of_Broadcasts_Requested from the AMF entity } then { the IUT sends a WRITE_REPLACE_WARNING_RESPONSE containing Message_Identifier, Serial_Number to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_WTP_02 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a WRITE-REPLACE WARNING REQUEST containing Message Identifier IE and/or Serial Number IE different from those in the warning message being broadcast and if Concurent Warning Message Indicator is not present then IUT node answers with WRITE-REPLACE WARNING RESPONSE and replaces the warning message being broadcast with newly received one for that area. |
| Reference | ETSI TS 138 413 [1], clauses 8.9.1.2 and 9.2.8.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/8_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a WRITE_REPLACE_WARNING_REQUEST containing Message_Identifier indicating value "different from already being broadcasted", Serial_Number indicating value "different from already being broadcasted", Repetition_Period, Number_of_Broadcasts_Requested, not Concurent_Warning_Message_Indicator from the AMF entity } then { the IUT sends a WRITE_REPLACE_WARNING_RESPONSE containing Message_Identifier, Serial_Number to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_WTP_03 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully processes a PWS CANCEL REQUEST containing mandatory IEs and answers with PWS CANCEL RESPONSE to cancel an already ongoing broadcast warning messages in all of the cells in the NG-RAN. |
| Reference | ETSI TS 138 413 [1], clauses 8.9.2.2 and 9.2.8.4 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/8_2 |
| Initial Conditions |  |
| with { the UE isNotRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PWS_CANCEL_REQUEST containing Message_Identifier, Serial_Number from the AMF entity } then { the IUT sends a PWS_CANCEL_RESPONSE containing Message_Identifier, Serial_Number to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_WTP_04 |
| --- | --- |
| Test Objective | Verify that the IUT can send a PWS RESTART INDICATION to inform AMF that PWS information for some or all cells may be reloaded from the CBC if needed. |
| Reference | ETSI TS 138 413 [1], clauses 8.9.3.2 and 9.2.8.5 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/8_3 |
| Initial Conditions |  |
| with { the UE isNotRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a PWS restart indication } then { the IUT sends a PWS_RESTART_INDICATION containing Cell_List_for_Restart, Global_RAN_Node_ID, TAI_List_for_Restart, Emergency_Area_ID_List_for_Restart to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_WTP_05 |
| --- | --- |
| Test Objective | Verify that the IUT can send a PWS FAILURE INDICATION to inform AMF that ongoing PWS operation for one or more cells has failed. |
| Reference | ETSI TS 138 413 [1], clauses 8.9.4.2 and 9.2.8.6 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/8_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a PWS failure indication } then { the IUT sends a PWS_FAILURE_INDICATION containing PWS_Failed_Cell_List, Global_RAN_Node_ID |  |

to the AMF entity

}

}

##### NRPPa Transport Procedures

| TP Id | TP_NGAP_GNB_NTP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send an UPLINK_UE_ASSOCIATED_NRPPA_TRANSPORT to carry NRPPA signaling between NG-RAN and LMF(Location Management Functionality). |
| Reference | ETSI TS 138 413 [1], clauses 8.10.2.2 and 9.2.9.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/9_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate an NRPPA transport procedure } then { the IUT sends an UPLINK_UE_ASSOCIATED_NRPPA_TRANSPORT containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Routing_ID, NRPPa_PDU to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_NTP_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send an UPLINK_NON_UE_ASSOCIATED_NRPPA_TRANSPORT to carry NRPPA signaling between NG-RAN and LMF(Location Management Functionality). |
| Reference | ETSI TS 138 413 [1], clauses 8.10.2.4 and 9.2.9.4 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/9_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate an NRPPA transport procedure } then { the IUT sends an UPLINK_NON_UE_ASSOCIATED_NRPPA_TRANSPORT containing Routing_ID, NRPPa_PDU to the AMF entity } } |  |

##### Trace Procedures

| TP Id | TP_NGAP_GNB_TRP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a TRACE_FAILURE_INDICATION to inform AMF that Trace Start procedure has failed due to an interaction with handover procedure. |
| Reference | ETSI TS 138 413 [1], clauses 8.11.1.2, 8.11.2.2, 9.2.10.1 and 9.2.10.2 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/10_1 and PICS_A3/10_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a TRACE_START containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Trace_Activation containing NG_RAN_Trace_ID, Interfaces_to_Trace, Trace_Depth, Trace_Collection_Entity_IP_Address from the AMF entity } then { the IUT sends a TRACE_FAILURE_INDICATIONT containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, NG_RAN_Trace_ID, Cause to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_TRP_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send a TRACE_FAILURE_INDICATION to inform AMF that a Deactivate Trace procedure has failed due to an interaction with handover procedure. |
| Reference | ETSI TS 138 413 [1], clauses 8.11.2.2, 8.11.3.2, 9.2.10.2 and 9.2.10.3 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/10_2 and PICS_A3/10_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a DEACTIVATE_TRACE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, NG_RAN_Trace_ID from the AMF entity } then { the IUT sends a TRACE_FAILURE_INDICATIONT containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, NG_RAN_Trace_ID, Cause to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_TRP_03 |
| --- | --- |
| Test Objective | Verify that the IUT can send a CELL_TRAFFIC_TRACE to send the allocated Trace Recording Session Reference and Trace Reference to AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.11.4.2 and 9.2.10.4 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/10_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a cell traffic trace procedure } then { the IUT sends a CELL_TRAFFIC_TRACE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, NG_RAN_Trace_ID, |  |

NG_RAN_CGI,

Trace_Colection_Entity_IP_Address          to the AMF entity

}

}

##### Location Reporting Procedures

| TP Id | TP_NGAP_GNB_LRP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a LOCATION REPORTING FAILURE INDICATION message after that contains mandatory IEs receiving a LOCATION REPORTING CONTROL message with two identical Location Reporting Reference IDs. |
| Reference | ETSI TS 138 413 [1], clauses 8.12.2, 9.2.11.1, 9.2.11.2 and 9.3.1.65 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/11_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a LOCATION_REPORTING_CONTROL containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Location_Reporting_Request_Type containing Event_Type indicating value direct, Report_Area indicating value cell, Area_of_Interest_List containing Area_of_Interest_Item containing Area_of_Interest, Location_Reporting_Reference_ID indicating value PX_REPORTING_REFERENCE_ID, Area_of_Interest_Item containing Area_of_Interest, Location_Reporting_Reference_ID indicating value PX_REPORTING_REFERENCE_ID from the AMF entity } then { the IUT sends a LOCATION_REPORTING_FAILURE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Cause containing CHOICE_Cause_Group containing Radio_Network_Layer_Cause indicating value Multiple_Location_Reporting_Reference_ID_instances to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_LRP_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send a LOCATION REPORT message that contains mandatory IEs to provide the UE's current location. |
| Reference | ETSI TS 138 413 [1], clauses 8.12.3 and 9.2.11.3 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/11_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a LOCATION_REPORTING_CONTROL containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Location_Reporting_Request_Type containing Event_Type indicating value direct, Report_Area indicating value cell |  |

2 V1.1.1 (2024-10)

from the AMF entity } then { the IUT sends a LOCATION_REPORT containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, User_Location_Information, Location_Reporting_Request_Type to the AMF entity } }

##### UE TNLA Binding Procedures

Void

##### UE Radio Capability Management Procedures

| TP Id | TP_NGAP_GNB_URP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UE_RADIO_CAPABILITY_INFO_INDICATION. |
| Reference | ETSI TS 138 413 [1], clauses 8.14.1.2 and 9.2.13.1 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/13_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a UE Radio Capability Management procedure } then { the IUT sends a UE_RADIO_CAPABILITY_INFO_INDICATION containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, UE_Radio_Capability to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_URP_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UE RADIO CAPABILITY CHECK RESPONSE message that contains mandatory IEs to provide the UE radio capabilities. |
| Reference | ETSI TS 138 413 [1], clauses 8.14.2.2, 9.2.13.2 and 9.2.13.3 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/13_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a UE_RADIO_CAPABILITY_CHECK_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID from the AMF entity } then { the IUT sends a UE_RADIO_CAPABILITY_CHECK_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, IMS_Voice_Support_Indicator to the AMF entity } } |  |

| TP Id | TP_NGAP_GNB_URP_03 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UE RADIO CAPABILITY ID MAPPING REQUEST. |
| Reference | ETSI TS 138 413 [1], clauses 8.14.3.2 and 9.2.13.4 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/13_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a UE Radio Capability Management procedure } then { the IUT sends a UE_RADIO_CAPABILITY_ID_MAPPING_REQUEST containing UE_Radio_Capability_ID to the AMF entity } } |  |

##### Data Usage Reporting Procedures

| TP Id | TP_NGAP_GNB_DRP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a SECONDARY RAT DATA USAGE REPORT. |
| Reference | ETSI TS 138 413 [1], clauses 8.15.1.2 and 9.2.14.1 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/14_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a Data_Usage_Reporting_procedure } then { the IUT sends a SECONDARY_RAT_DATA_USAGE_REPORT containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Secondary_RAT_Usage_List containing PDU_Session_Resource_Secondary_RAT_Usage_Item containing PDU_Session_Id, Secondary_RAT_Data_Usage_Report_Transfer to the AMF entity } } |  |

##### RIM Information Transfer Procedures

| TP Id | TP_NGAP_GNB_RIP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UPLINK RIM INFORMATION TRANSFER. |
| Reference | ETSI TS 138 413 [1], clauses 8.16.1.2 and 9.2.15.1 |
| Configuration | CF_GNB_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/15_1 |
| Initial Conditions |  |
| with { the UE isNotRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a RIM Information Transfer procedure } then { the IUT sends a UPLINK_RIM_INFORMATION_TRANSFER to the AMF entity |  |

}

}

#### AMF Role

##### Test selection

The IUT takes the role of the AMF; PICS A.2/2.

##### PDU Session Management Procedures

| TP Id | TP_NGAP_AMF_PDU_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a PDU SESSION RESOURCE SETUP REQUEST with at least one PDU session resource list to established PDU session. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.1.2 and 9.2.1.1 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/1_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a PDU session management procedure } then { the IUT sends a PDU_SESSION_RESOURCE_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Setup_Request_List containing PDU_SessionId, S_NSSAI, PDU_Session_Resource_Setup_Request_Transfer to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_PDU_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send a PDU_SESSION_RESOURCE_RELEASE_COMMAND to release PDU session. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.1.2 and 9.2.1.3 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/1_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a PDU session resource release procedure } then { the IUT sends a PDU_SESSION_RESOURCE_RELEASE_COMMAND containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_To_Release_List containing PDU_Session_Resource_To_Release_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI, PDU_Session_Resource_Release_Command_Transfer containing Cause indicating value PX_Cause to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_PDU_03 |
| --- | --- |
| Test Objective | Verify that the IUT can send a PDU_SESSION_RESOURCE_MODIFY_REQUEST to modify PDU session. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.3.2 and 9.2.1.5 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/1_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a PDU session resource modify procedure } then { the IUT sends a PDU_SESSION_RESOURCE_MODIFY_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Modify_Request_List containing PDU_Session_Resource_Modify_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, QoSFlowAddorModifyRequestList containing QoSFlowAddorModifyRequestItem containing QoSFlowIdentifier to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_PDU_04 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes a PDU_SESSION_RESOURCE_MODIFY_INDICATION containing mandatory IEs and answers with PDU_SESSION_RESOURCE_MODIFY_CONFIRM for successfully modified PDU session. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.5.2 and 9.2.1.8 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/1_5 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PDU_SESSION_RESOURCE_MODIFY_INDICATION containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Modify_Indication_List containing PDU_Session_Resource_Modify_Indication_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Modify_Indication_Transfer containing DLQoSFlowperTNLInformation containing UPTransportLayerInformation, AssociatedQoSFlowList containing AssociatedQoSFlowItem containing QoSFlowIdentifier indicating value PX_QoS_Flow_ID from the GNB entity } then { the IUT sends a PDU_SESSION_RESOURCE_MODIFY_CONFIRM containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Modify_Confirm_List containing PDU_Session_Resource_Modify_Confirm_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Modify_Confirm_Transfer containing QoSFlowModifyConfirmList containing QoSFlowModifyConfirmItem containing QoSFlowIdentifier, UL_NG_U_UP_TNLInformation to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_PDU_05 |
| --- | --- |
| Test Objective | Verify that the AMF node processes a PDU_SESSION_RESOURCE_MODIFY_INDICATION containing mandatory IEs and answers with PDU_SESSION_RESOURCE_MODIFY_CONFIRM for not successfully modified PDU session. |
| Reference | ETSI TS 138 413 [1], clauses 8.2.5.2 and 9.2.1.8 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/1_5 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PDU_SESSION_RESOURCE_MODIFY_INDICATION containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Modify_Indication_List containing PDU_Session_Resource_Modify_Indication_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Modify_Indication_Transfer containing DLQoSFlowperTNLInformation containing UPTransportLayerInformation, AssociatedQoSFlowList containing AssociatedQoSFlowItem containing QoSFlowIdentifier indicating value PX_QoS_Wrong_Flow_ID from the GNB entity } then { the IUT sends a PDU_SESSION_RESOURCE_MODIFY_CONFIRM containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Failed_To_Modify_List containing PDU_Session_Resource_Failed_To_Modify_Item containing PDU_SessionId indicating value PX_PDU_ID, PDU_Session_Resource_Modify_Indication_Unsuccessful_Transfer containing Cause to the GNB entity } } |  |

##### UE Context Management Procedures

| TP Id | TP_NGAP_AMF_CMP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send an INITIAL CONTEXT SETUP REQUEST containing mandatory IEs. |
| Reference | ETSI TS 138 413 [1], clause 8.3.1.2 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/2_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a PDU initial context setup procedure } then { the IUT sends an INITIAL_CONTEXT_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, PDU_Session_Resource_Setup_Request_List containing PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, PDU_Session_Resource_Setup_Request_Transfer, Allowed_NSSAI containing |  |

Allowed_NSSAI_List containing

Allowed_NSSAI_Item containing

S_NSSAI containing

SST,

UE_Security_Capabilities containing

NR_Encryption_Algorithms,

NR_Integrity_Protection_Algorithms,

E_UTRA_Encryption_Algorithms,

E_UTRA_Integrity_Protection_Algorithms,                             Security_Key                         to the GNB entity

}

}

| TP Id | TP_NGAP_AMF_CMP_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UE CONTEXT RELEASE COMMAND that contains both the AMF UE NGAP ID IE and the RAN UE NGAP ID IE. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.3 and 9.2.2.5 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/2_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a UE context release command } then { the IUT sends a UE_CONTEXT_RELEASE_COMMAND containing CHOICE_UE_NGAP_IDs containing UE_NGAP_ID_pair containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Cause to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_CMP_03 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UE CONTEXT RELEASE COMMAND where both the AMF UE NGAP ID IE and the RAN UE NGAP ID IE are not available. |
| Reference | ETSI TS 138 413 [1], clause 8.3.3 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/2_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a UE context release command } then { the IUT sends a UE_CONTEXT_RELEASE_COMMAND containing CHOICE_UE_NGAP_IDs containing AMF_UE_NGAP_ID containing AMF_UE_NGAP_ID, Cause to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_CMP_04 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UE CONTEXT MODIFICATION REQUEST containing mandatory IEs. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.4.2 and 9.2.2.7 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/2_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a PDU UE context modification procedure } then { the IUT sends a UE_CONTEXT_MODIFICATION_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_CMP_05 |
| --- | --- |
| Test Objective | Verify that the IUT can send a CONNECTION ESTABLISHMENT INDICATION that contains mandatory IEs. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.6.2 and 9.2.2.11 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/2_6 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a PDU UE context modification procedure } then { the IUT sends a CONNECTION_ESTABLISHMENT_INDICATION containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_CMP_06 |
| --- | --- |
| Test Objective | Verify that the IUT can send an AMF CP RELOCATION INDICATION that contains mandatory IEs. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.7.2 and 9.2.2.12 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/2_7 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate an AMF CP relocation indication procedure } then { the IUT sends an AMF_CP_RELOCATION_INDICATION containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the GNB entity } } |  |
| TP Id | TP_NGAP_AMF_CMP_07 |
| Test Objective | Verify that the IUT can successfully request UE information from the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.9.2, 8.3.10.2, 9.2.2.14 and 9.2.2.15 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/2_9 and PICS_A4/2_10 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a RETRIEVE_UE_INFORMATION containing 5G_S_TMSI containing AMF_Set_ID, AMF_Pointer, 5G_TMSI from the GNB entity } then { the IUT sends a UE_INFORMATION_TRANSFER containing 5G_S_TMSI containing AMF_Set_ID, AMF_Pointer, 5G_TMSI to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_CMP_08 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes a UE CONTEXT SUSPEND REQUEST containing mandatory IEs and answers with UE CONTEXT SUSPEND REQUEST RESPONSE with successfully suspend UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.11.2, 9.2.2.16 and 9.2.2.17 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/2_11 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a UE_CONTEXT_SUSPEND_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Suspend_List containing PDU_Session_Resource_Suspend_Item containing PDU_SessionId indicating value PX_PDU_ID, UE_Context_Suspend_Request_Transfer containing Suspend_Indicator from the GNB entity } then { the IUT sends a UE_CONTEXT_SUSPEND_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_CMP_09 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes a UE CONTEXT SUSPEND REQUEST containing mandatory IEs and answers with UE CONTEXT SUSPEND FAILURE with failed suspension. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.11.3, 9.2.2.16 and 9.2.2.18 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/2_11 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext and the UE hasPendingDataTransmission } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a UE_CONTEXT_SUSPEND_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Suspend_List containing PDU_Session_Resource_Suspend_Item containing PDU_SessionId indicating value PX_PDU_ID, UE_Context_Suspend_Request_Transfer containing Suspend_Indicator from the GNB entity } then { the IUT sends a UE_CONTEXT_SUSPEND_FAILURE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Cause to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_CMP_10 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes a UE CONTEXT RESUME REQUEST containing mandatory IEs and answers with UE CONTEXT RESUME RESPONSE with successfully resume UE context. |
| Reference | ETSI TS 138 413 [1], clauses 8.3.12.3, 9.2.2.19 and 9.2.2.20 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/2_12 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasSuspendedContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a UE_CONTEXT_RESUME_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, RRC_Resume_Cause from the GNB entity } then { the IUT sends a UE_CONTEXT_RESUME_RESPONSE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_CMP_11 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes a UE CONTEXT RESUME REQUEST containing mandatory IEs and answers with UE CONTEXT RESUME FAILURE with failing to resume UE context. |
| Reference | ETSI TS 138 413 [1], clause 8.3.12.3 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/2_12 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasSuspendedContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a UE_CONTEXT_RESUME_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, RRC_Resume_Cause, PDU_Session_Resource_Resume_List containing PDU_Session_Resource_Resume_Item containing PDU_SessionId indicating value PX_PDU_ID, UE_Context_Resume_Request_Transfer from the GNB entity } then { the IUT sends a UE_CONTEXT_RESUME_FAILURE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Cause to the GNB entity } } |  |

##### UE Mobility Management

| TP Id | TP_NGAP_AMF_MMP_01 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes a HANDOVER REQUIRED message containing mandatory IEs and answers with HANDOVER COMMAND with successfully handover. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.1.2, 9.2.3.1 and 9.2.3.2 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/3_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a HANDOVER_REQUIRED containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Handover_Typ, Cause, Target_ID, PDU_Session_Resource_List containing PDU_Session_Resource_Item containing PDU_SessionId indicating value PX_PDU_ID, Handover_Required_Transfer, Source_to_Target_Transparent_Container from the GNB entity } then { the IUT sends a HANDOVER_COMMAND containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Handover_Type, PDU_Session_Resource_Handover_List containing PDU_Session_Resource_Handover_Item containing PDU_SessionId indicating value PX_PDU_ID, Handover_Command_Transfer to the GNB entity |  |

}

}

| TP Id | TP_NGAP_AMF_MMP_02 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes a HANDOVER REQUIRED message containing mandatory IEs and answers with HANDOVER PREPARATION FAILURE because the UE is not longer available. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.1.3, 9.2.3.1 and 9.2.3.3 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/3_1 |
| Initial Conditions |  |
| with { the UE isNoLongerAvailable } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a HANDOVER_REQUIRED containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Handover_Typ, Cause, Target_ID, PDU_Session_Resource_List containing PDU_Session_Resource_Item containing PDU_SessionId, Handover_Required_Transfer, Source_to_Target_Transparent_Container from the GNB entity } then { the IUT sends a HANDOVER_PREPARATION_FAILURE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Cause to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_MMP_03 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully sends a HANDOVER REQUEST message that contains mandatory IEs to the GNB. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.2.2 and 9.2.3.4 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/3_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a Handover Request procedure } then { the IUT sends a HANDOVER_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Handover_Typ, Cause, UE_Aggregate_Maximum_Bit_Rate containing UE_Aggregate_Maximum_Bit_Rate_Downlink, UE_Aggregate_Maximum_Bit_Rate_Uplink, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, PDU_Session_Setup_Resource_List containing PDU_Session_Setup_Resource_Item containing PDU_SessionId, S_NSSAI containing |  |
| SST, Handover_Request_Transfer, Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing S_NSSAI containing SST, Source_to_Target_Transparent_Container, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_MMP_04 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes a PATH SWITCH REQUEST message that contains mandatory IEs and answers with PATH SWITCH REQUEST ACKNOWLEDGE to acknowledge the path switch. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.4.2, 9.2.3.8 and 9.2.3.9 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/3_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PATH_SWITCH_REQUEST containing RAN_UE_NGAP_ID, Source_AMF_UE_NGAP_ID, User_Location_Information, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, PDU_Session_Resources_to_be_Switched_in_Downlink_List containing PDU_Session_Resources_to_be_Switched_in_Downlink_Item containing PDU_SessionId, Path_Switch_Request_Transfer from the GNB entity } then { the IUT sends a PATH_SWITCH_REQUEST_ACKNOWLEDGE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Security_Context containing Next_Hop_Chaining_Count, Next_Hop_NH , PDU_Session_Resource_Switched_List containing PDU_Session_Resource_Switched_Item containing PDU_SessionId, Path_Switch_Request_Acknowledge_Transfer to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_MMP_05 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes a PATH SWITCH REQUEST message that contains mandatory IEs and two PDU Sessions with same ID and answers with PATH SWITCH REQUEST FAILURE. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.4.4, 9.2.3.8 and 9.2.3.10 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/3_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE hasEstablishedInitialContext } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a PATH_SWITCH_REQUEST containing RAN_UE_NGAP_ID, Source_AMF_UE_NGAP_ID, User_Location_Information, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, PDU_Session_Resources_to_be_Switched_in_Downlink_List containing PDU_Session_Resources_to_be_Switched_in_Downlink_Item containing PDU_SessionId, Path_Switch_Request_Transfer, PDU_Session_Resources_to_be_Switched_in_Downlink_Item containing PDU_SessionId, Path_Switch_Request_Transfer from the GNB entity } then { the IUT sends a PATH_SWITCH_REQUEST_FAILURE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, PDU_Session_Resource_Released_List containing PDU_Session_Resource_Released_Item containing PDU_SessionId, Path_Switch_Request_Unsuccessful_Transfer containing Cause to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_MMP_06 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes a HANDOVER CANCEL message that contains mandatory IEs and answers with HANDOVER CANCEL ACKNOWLEDGE to confirm that the ongoing handover was cancelled. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.5.2, 9.2.3.11 and 9.2.3.12 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/3_5 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the GNB alreadyPreparedHandover } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a HANDOVER_CANCEL containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Cause from the GNB entity } then { the IUT sends a HANDOVER_CANCEL_ACKNOWLEDGE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the GNB entity |  |

}

}

| TP Id | TP_NGAP_AMF_MMP_07 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully sends a DOWNLINK RAN STATUS TRANSFER message that contains mandatory IEs. |
| Reference | ETSI TS 138 413 [1], clause 8.4.6.2 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/3_6 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the GNB alreadyPreparedHandover } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a Downlink RAN Status Transfer procedure } then { the IUT sends a DOWNLINK_RAN_STATUS_TRANSFER containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, RAN_Status_Transfer_Transparent_Container containing DRBs_Subject_To_Status_Transfer_List containing DRBs_Subject_To_Status_Transfer_Item containing DRB_ID, CHOICE_UL_DRB_Status containing 12_bits containing UL_COUNT_Value containing PDCP_SN_Length_12, HFN_for_PDCP_SN_Length_12, CHOICE_DL_DRB_Status containing 12_bits containing DL_COUNT_Value containing PDCP_SN_Length_12, HFN_for_PDCP_SN_Length_12 to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_MMP_08 |
| --- | --- |
| Test Objective | Verify that the NG-RAN node successfully sends a Handover Success message to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.4.8.2 and 9.2.3.15 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/1 and PICS_A3/3_8 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the GNB completedHandover } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of a UE handover success procedure" } then { the IUT sends a HANDOVER_SUCCESS containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_MMP_09 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully sends a DOWNLINK RAN EARLY STATUS TRANSFER message that contains mandatory IEs. |
| Reference | ETSI TS 138 413 [1], clause 8.4.9.2 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/3_9 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the GNB alreadyPreparedHandover } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a Downlink RAN Early Status Transfer procedure } then { the IUT sends a DOWNLINK_RAN_EARLY_STATUS_TRANSFER containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Early_Status_Transfer_Transparent_Container containing CHOICE_Procedure_Stage containing First_DL_COUNT containing DRBs_Subject_To_Early_Status_Transfer_List containing DRBs_Subject_To_Early_Status_Transfer_Item containing DRB_ID, CHOICE_First_DL_COUNT containing 12_bits containing FIRST_DL_COUNT_Value containing PDCP_SN_Length_12, HFN_for_PDCP_SN_Length_12 to the GNB entity } } |  |

##### Paging Procedures

| TP Id | TP_NGAP_AMF_PAG_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a PAGING message to enable the AMF to page a UE. |
| Reference | ETSI TS 138 413 [1], clauses 8.5.1.2 and 9.2.4.1 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/4_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of a Paging procedure" } then { the IUT sends a PAGING containing UE_Paging_Identity containing 5G_S_TMSI containing 5G_S_TMSI containing AMF_Set_ID, AMF_Pointer, 5G_TMSI, TAI_List_for_Paging containing TAI_List_for_Paging_Item containing TAI containing PLMN_Identity, TAC to the GNB entity } } |  |

##### Transport of NAS Messages Procedures

| TP Id | TP_NGAP_AMF_NAS_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a DOWNLINK NAS TRANSPORT message to carry NAS information over the NG interface. |
| Reference | ETSI TS 138 413 [1], clauses 8.6.2.2, 9.2.5.1 and 9.2.5.2 ETSI TS 123 502 [3], clause 4.24.1 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/5_2 |
| Initial Conditions |  |
| with { the UE isCMCONNECTED and the GNB hasReceivedNASMessage from the UE entity } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an INITIAL_UE_MESSAGE containing RAN_UE_NGAP_ID, NAS_PDU, User_Location_Information, RRC_Establishmnet_Cause from the GNB entity } then { the IUT sends a DOWNLINK_NAS_TRANSPORT containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, NAS_PDU to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_NAS_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send a REROUTE NAS REQUEST message to reroute the INITIAL UE MESSAGE message to another AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.6.5.2 and 9.2.5.5 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/5_5 |
| Initial Conditions |  |
| with { the AMF isNotAppropriateToServe the UE } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an INITIAL_UE_MESSAGE containing RAN_UE_NGAP_ID, NAS_PDU, User_Location_Information, RRC_Establishmnet_Cause from the GNB entity } then { the IUT sends a REROUTE_NAS_REQUEST containing RAN_UE_NGAP_ID, NGAP_Message, // Contains the INITIAL UE MESSAGE NAS_PDU, AMF_Set_ID to the GNB entity } } |  |

##### Interface Management Procedures

| TP Id | TP_NGAP_AMF_IMP_01 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes an NG SETUP REQEST message that contains mandatory IEs and answers with NG SETUP RESPONSE to acknowledge the setup. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.1.2, 9.2.6.1 and 9.2.6.2 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_1 |
| Initial Conditions |  |
| with { the UE hasEstablishedRRCConnection } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an NG_SETUP_REQUEST containing Global_RAN_Node_ID, Supported_TA_List containing Supported_TA_Item containing TAC, Broadcast_PLMN_List containing Broadcast_PLMN_Item containing PLMN_Identity, TAI_Slice_Support_List containing S_NSSAI containing SST, Default_Paging_DRX from the GNB entity } then { the IUT sends an NG_SETUP_RESPONSE containing AMF_Name, Served_GUAMI_List containing Served_GUAMI_Item containing GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, Relative_AMF_Capacity, PLMN_Support_List containing PLMN_Support_Item containing PLMN_Identity, Slice_Support_List containing S_NSSAI containing SST, Extended_Slice_Support_List containing S_NSSAI containing SST to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_IMP_02 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully decline an NG RESET REQEST message and answers with NG SETUP FAILURE when the AMF is not able to handle the request. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.1.3, 9.2.6.1 and 9.2.6.3 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_1 |
| Initial Conditions |  |
| with { the UE hasEstablishedRRCConnection } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an NG_SETUP_REQUEST containing Global_RAN_Node_ID, Supported_TA_List containing Supported_TA_Item containing TAC, Broadcast_PLMN_List containing Broadcast_PLMN_Item containing |  |

2 V1.1.1 (2024-10)

PLMN_Identity, TAI_Slice_Support_List containing S_NSSAI containing SST, Default_Paging_DRX from the GNB entity } then { the IUT sends an NG_SETUP_FAILURE containing Cause to the GNB entity } }

| TP Id | TP_NGAP_AMF_IMP_03 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes an NG SETUP REQEST message with optional field UE Retention Information and answers with NG SETUP RESPONSE to acknowledge the setup. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.1.2, 9.2.6.1 and 9.2.6.2 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_1 |
| Initial Conditions |  |
| with { the UE hasEstablishedRRCConnection } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an NG_SETUP_REQUEST containing Global_RAN_Node_ID, Supported_TA_List containing Supported_TA_Item containing TAC, Broadcast_PLMN_List containing Broadcast_PLMN_Item containing PLMN_Identity, TAI_Slice_Support_List containing S_NSSAI containing SST, Default_Paging_DRX, UE_Retention_Information from the GNB entity } then { the IUT sends an NG_SETUP_RESPONSE containing AMF_Name, Served_GUAMI_List containing Served_GUAMI_Item containing GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, Relative_AMF_Capacity, PLMN_Support_List containing PLMN_Support_Item containing PLMN_Identity, Slice_Support_List containing S_NSSAI containing SST, Extended_Slice_Support_List containing S_NSSAI containing SST, UE_Retention_Information to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_IMP_04 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes an NG SETUP REQEST message with different optional fields and answers with NG SETUP RESPONSE to acknowledge the setup. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.1.2, 9.2.6.1 and 9.2.6.2 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_1 |
| Initial Conditions |  |
| with { the UE hasEstablishedRRCConnection } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an NG_SETUP_REQUEST containing Global_RAN_Node_ID, Supported_TA_List containing Supported_TA_Item containing TAC Broadcast_PLMN_List containing Broadcast_PLMN_Item containing PLMN_Identity, TAI_Slice_Support_List containing S_NSSAI containing SST, NPN_Support, Extended_TAI_Slice_Support_List containing S_NSSAI containing SST, Configured_TAC_Indication, RAT_Information, Default_Paging_DRX, NB_IoT_Default_Paging_DRX, Extended_RAN_Node_Name containing RAN_Node_Name_Visible from the GNB entity } then { the IUT sends an NG_SETUP_RESPONSE containing AMF_Name, Served_GUAMI_List containing Served_GUAMI_Item containing GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, Relative_AMF_Capacity, PLMN_Support_List containing PLMN_Support_Item containing PLMN_Identity, Slice_Support_List containing S_NSSAI containing SST, NPN_Support, Extended_Slice_Support_List containing S_NSSAI containing SST, Extended_AMFName to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_IMP_05 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes a RAN CONFIGURATION UPDATE message with RAN CONFIGURATION UPDATE ACKNOWLEDGE to acknowledge the update. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.2.2, 9.2.6.4 and 9.2.6.5 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a RAN_CONFIGURATION_UPDATE containing Supported_TA_List containing Supported_TA_Item containing TAC, Broadcast_PLMN_List containing Broadcast_PLMN_Item containing PLMN_Identity, TAI_Slice_Support_List containing S_NSSAI containing SST from the GNB entity } then { the IUT sends a RAN_CONFIGURATION_UPDATE_ACKNOWLEDGE to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_IMP_06 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully declines a RAN CONFIGURATION UPDATE message with RAN CONFIGURATION UPDATE FAILURE when the AMF is not able to handle the request. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.2.2, 9.2.6.4 and 9.2.6.6 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a RAN_CONFIGURATION_UPDATE containing Supported_TA_List containing Supported_TA_Item containing TAC, Broadcast_PLMN_List containing Broadcast_PLMN_Item containing PLMN_Identity, TAI_Slice_Support_List containing S_NSSAI containing SST from the GNB entity } then { the IUT sends a RAN_CONFIGURATION_UPDATE_FAILURE containing Cause to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_IMP_07 |
| --- | --- |
| Test Objective | Verify that the IUT can send an AMF CONFIGURATION UPDATE message to the AMF. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.3.2 and 9.2.6.7 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE isCMIDLE and the UE hasReceivedPAGINGMessage } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of an AMF CONFIGURATION UPDATE procedure" } then { the IUT sends an AMF_CONFIGURATION_UPDATE containing Served_GUAMI_List containing |  |
| Served_GUAMI_Item containing GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, PLMN_Support_List containing PLMN_Support_Item containing PLMN_Identity, Slice_Support_List containing S_NSSAI containing SST to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_IMP_08 |
| --- | --- |
| Test Objective | Verify that the AMF node successfully processes an NG RESET message that contains mandatory IEs and answers with NG RESET ACKNOWLEDGE to acknowledge the reset. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.4.2.2, 9.2.6.11 and 9.2.6.12 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_4 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF and the UE isRequestedToDetachfromNetwork } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an NG_RESET containing Cause, CHOICE_Reset_Type containing NG_interface containing Reset_All from the GNB entity } then { the IUT sends an NG_RESET_ACKNOWLEDGE to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_IMP_09 |
| --- | --- |
| Test Objective | Verify that the AMF can send an ERROR INDICATION to the GNB when a error occurs. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.5.2 and 9.2.6.13 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_5 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives an INITIAL_CONTEXT_SETUP_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer, PDU_Session_Resource_Setup_Request_List containing PDU_Session_Resource_Setup_Request_Item containing PDU_SessionId indicating value PX_PDU_ID, S_NSSAI containing SST, PDU_Session_Resource_Setup_Request_Transfer, Allowed_NSSAI containing Allowed_NSSAI_List containing Allowed_NSSAI_Item containing |  |
| S_NSSAI containing SST, UE_Security_Capabilities containing NR_Encryption_Algorithms, NR_Integrity_Protection_Algorithms, E_UTRA_Encryption_Algorithms, E_UTRA_Integrity_Protection_Algorithms, Security_Key from the GNB entity } then { the IUT sends an ERROR_INDICATION to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_IMP_10 |
| --- | --- |
| Test Objective | Verify that the AMF can send an AMF STATUS INDICATION to the GNB when a error occurs. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.6.2 and 9.2.6.10 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_6 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate an AMF STATUS INDICATION } then { the IUT sends an AMF_STATUS_INDICATION containing Unavailable_GUAMI_List containing Unavailable_GUAMI_Item containing GUAMI containing PLMN_Identity, AMF_Region_ID, AMF_Set_ID, AMF_Pointer to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_IMP_11 |
| --- | --- |
| Test Objective | Verify that the AMF can send an OVERLOAD START to the GNB. |
| Reference | ETSI TS 138 413 [1], clause 8.7.7.2 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_7 |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate an OVERLOAD START } then { the IUT sends an OVERLOAD_START containing Overload_Start_NSSAI_List containing Overload_Start_NSSAI_Item containing Slice_Overload_List containing S_NSSAI containing SST to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_IMP_12 |
| --- | --- |
| Test Objective | Verify that the AMF can send an OVERLOAD STOP to the GNB. |
| Reference | ETSI TS 138 413 [1], clauses 8.7.8.2 and 9.2.6.15 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/6_8 |
| Initial Conditions |  |
| with { the GNB hasReceivedOVERLOADSTARTtMessage } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate an OVERLOAD STOP } then { the IUT sends an OVERLOAD_STOP to the GNB entity } } |  |

##### Configuration Transfer Procedure

| TP Id | TP_NGAP_AMF_CTP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a DOWNLINK RAN CONFIGURATION TRANSFER message to transfer RAN configuration information to the NG-RAN. |
| Reference | ETSI TS 138 413 [1], clauses 8.8.2 and 9.2.7.2 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/7_2 |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of a Downlink RAN Configuration Transfer" } then { the IUT sends a DOWNLINK_RAN_CONFIGURATION_TRANSFER to the GNB entity } } |  |

##### Warning Message Transmission Procedures

| TP Id | TP_NGAP_AMF_WTP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a WRITE REPLACE WARNING REQUEST to start broadcasting of warning messages. |
| Reference | ETSI TS 138 413 [1], clauses 8.9.1.2 and 9.2.8.1 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/8_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a Write message Transmission procedure } then { the IUT sends a WRITE_REPLACE_WARNING_REQUEST containing Message_Identifier, Serial_Number, Repetition_Period, Number_of_Broadcasts_Requested to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_WTP_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send a PWS CANCEL REQUEST to cancel broadcasting of warning messages. |
| Reference | ETSI TS 138 413 [1], clauses 8.9.2.2 and 9.2.8.3 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/8_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a PWS cancel procedure } then { the IUT sends a PWS_CANCEL_REQUEST containing Message_Identifier, Serial_Number to the GNB entity } } |  |

##### NRPPa Transport

| TP Id | TP_NGAP_AMF_NTP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a DOWNLINK UE ASSOCIATED NRPPA TRANSPORT to carry NRPPA signaling between NG-RAN and LMF(Location Management Functionality). |
| Reference | ETSI TS 138 413 [1], clauses 8.10.2.1 and 9.2.9.1 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/9_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate an NRPPA transport procedure } then { the IUT sends a DOWNLINK_UE_ASSOCIATED_NRPPA_TRANSPORT containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Routing_ID, NRPPa_PDU to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_NTP_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send a DOWNLINK_NON_UE_ASSOCIATED_NRPPA_TRANSPORT to carry NRPPA signaling between NG-RAN and LMF(Location Management Functionality). |
| Reference | ETSI TS 138 413 [1], clauses 8.10.2.3 and 9.2.9.3 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/9_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate an NRPPA transport procedure } then { the IUT sends a DOWNLINK_NON_UE_ASSOCIATED_NRPPA_TRANSPORT containing Routing_ID, NRPPa_PDU to the GNB entity |  |

2 V1.1.1 (2024

}

}

##### Trace Procedures

| TP Id | TP_NGAP_AMF_TRP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a TRACE_START to initiate a trace session for a UE. |
| Reference | ETSI TS 138 413 [1], clauses 8.11.1.2 and 9.2.10.1 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/10_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a trace procedure } then { the IUT sends a TRACE_START containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Trace_Activation containing NG_RAN_Trace_ID, Interfaces_to_Trace, Trace_Depth, Trace_Collection_Entity_IP_Address to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_TRP_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send a DEACTIVATE_TRACE to deactivate a trace session for a UE. |
| Reference | ETSI TS 138 413 [1], clauses 8.11.3.2 and 9.2.10.3 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/10_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a deactivate trace procedure } then { the IUT sends a DEACTIVATE_TRACE containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, NG_RAN_Trace_ID to the GNB entity } } |  |

##### Location Reporting

| TP Id | TP_NGAP_AMF_LRP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a LOCATION REPORTING CONTROL message that contains mandatory IEs to request a report of the UE's current location. |
| Reference | ETSI TS 138 413 [1], clauses 8.12.1 and 9.2.11.1 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/11_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate the initiation "of a Location Reporting Control procedure" } then { the IUT sends a LOCATION_REPORTING_CONTROL containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID, Location_Reporting_Request_Type to the GNB entity } } |  |

##### UE TNLA Binding Procedures

| TP Id | TP_NGAP_AMF_UBP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UE TNLA BINDING RELEASE REQUEST to initiate a UE TNLA Binding Release procedure. |
| Reference | ETSI TS 138 413 [1], clauses 8.13.1.2 and 9.2.12.1 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/12_1 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a UE TNLA BINDING procedure } then { the IUT sends a UE_TNLA_BINDING_RELEASE_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the GNB entity } } |  |

##### UE Radio Capability Management

| TP Id | TP_NGAP_AMF_URP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UE RADIO CAPABILITY CHECK REQUEST to initiate UE Radio Capability Management procedure. |
| Reference | ETSI TS 138 413 [1], clauses 8.14.2.2 and 9.2.13.2 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/13_2 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a UE Radio Capability Management procedure } then { the IUT sends a UE_RADIO_CAPABILITY_CHECK_REQUEST containing AMF_UE_NGAP_ID, RAN_UE_NGAP_ID to the GNB entity } } |  |

| TP Id | TP_NGAP_AMF_URP_02 |
| --- | --- |
| Test Objective | Verify that the IUT can send a UE RADIO CAPABILITY ID MAPPING RESPONSE message that contains mandatory IEs to provide the UE radio capabilities id mapping. |
| Reference | ETSI TS 138 413 [1], clauses 8.14.3.2, 9.2.13.4 and 9.2.13.5 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/13_3 |
| Initial Conditions |  |
| with { the UE isRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT receives a UE_RADIO_CAPABILITY_ID_MAPPING_REQUEST containing UE_Radio_Capability_ID from the GNB entity } then { the IUT sends a UE_RADIO_CAPABILITY_ID_MAPPING_RESPONSE containing UE_Radio_Capability_ID, UE_Radio_Capability to the GNB entity } } |  |

##### Data Usage Reporting Procedures

Void.

##### RIM Information Transfer Procedures

| TP Id | TP_NGAP_AMF_RIP_01 |
| --- | --- |
| Test Objective | Verify that the IUT can send a DOWNLINK RIM INFORMATION TRANSFER. |
| Reference | ETSI TS 138 413 [1], clauses 8.16.2.2 and 9.2.15.2 |
| Configuration | CF_AMF_N2 |
| PICS Selection | PICS_A2/2 and PICS_A4/15_2 |
| Initial Conditions |  |
| with { the UE isNotRegisteredTo the AMF } |  |
| Expected Behaviour |  |
| ensure that { when { the IUT indicate a RIM Information Transfer procedure } then { the IUT sends a DOWNLINK_RIM_INFORMATION_TRANSFER to the GNB entity } } |  |

# Annex A (normative): TDL-TO source files

Each TP in clause 5 above has been written in TDL-TO and thus in a structured manner which is consistent with all other TPs. The TDL-TO text files for all test purposes are released in the ETSI forge repository:

• 	https://forge.etsi.org/rep/int/5g-core/ngap/-/tree/main/test_purposes.

# History

|  |  | Document history |
| --- | --- | --- |
| V1.1.1 | October 2024 | Publication |